# ADVANCED FALL&VITALS ALERT SYSTEM

PROJECT REPORT

MALAVIKA KR 25BOE10137

# INTRODUCTION

This project outlines the development of a wearable, real-time patient monitoring system designed to automatically detect and respond to medical emergencies, specifically focusing on patient falls.

The system utilizes an accelerometer to constantly analyze the

patient's motion and a heart rate sensor to assess their physiological state. The core objective is to minimize the time between a critical event (a fall) and the commencement of emergency aid.

# PROBLEM STATEMENT

The core problem is the critical delay and poor reliability inherent in current fall alert systems for at-risk individuals.

Falls are major injury causes, but existing systems either

fail to notify if the patient is incapacitated (the long-lie problem) or generate too many false alarms, wasting caregiver time.

# FUNCTIONAL REQUIREMENTS

Detect Impact: Trigger on >=2.5G

Confirm Fall: >=10 seconds stillness (=<0.2G). Check Pulse: Wait 3s. Alert if < 50 or >20 BPM.

Notify: Send alert on confirmation/

critical pulse

set is_patient_safe to FALSE.

# NON FUNCTIONAL REQUIREMENTS

Performance: Real-time data processing (< 100 ms latency).

Reliability: High accuracy (>= 95% TPR, =<1% FPR).

Availability: High uptime (99.9%).

Security: Data encryption (HIPAA/GDPR) and access authentication.

Constraint: Battery life >=24 hours.

# SYSTEM ARCHITECTURE

Wearable: Sensors (Accel + Pulse) to MCU (Logic >= 2.5G /10sStillness).

Gateway: Relays Wearable data (BLE) to Cloud (Wi-Fi/Cellular).

Cloud Server: Validates Alerts, Logs Data, Sends Notifications.

UI: Caregiver Dashboard for Alerts and Acknowledgment.

# DESIGN DIAGRAMS

IMPLEMENTATION DETAILS

Fall Confirmation (Accelerometer): A high impact followed by

10 seconds of stillness  confirms the fall and triggers a primary alert.

Condition Check (Pulse): 3seconds after the fall, the patient's heart rate is checked. An alert is critically

escalated if the pulse is outside the 50 to 120 BPM

# TESTING APPROACH

Unit Testing: Verify individual constants work Integration Testing: Test the full logic flow using simulated data.

System Testing: Test real-world reliability using physical scenarios

(deliberate falls, ADL false alarms) and check alert latency

# CHALLENGES FACED

False Alarms: High rate of False Positives (e.g., sitting heavily) and False Negatives (missing slow falls).

Calibration: Fixed thresholds (2.5G) don't adapt to individual patients

or environments.

Power: Continuous monitoring of G-force and pulse is a major battery drain.

Data Quality: Motion artifacts corrupt heart rate data after a fall.

Compliance: Device must be comfortable for 24/7 wear to be effective.

# LEARNING AND TAKEAWAY

Learning: Simple, static thresholds (like 2.5G for impact) are efficient but fundamentally limited. They cannot distinguish between a real, dangerous fall and vigorous everyday activities.

Takeaway: Robust safety systems require adaptive algorithms or

machine learning to minimize false alarms (reducing caregiver "alarm fatigue") while ensuring high sensitivity to genuine falls.

# FUTURE ENHANCEMENTS

AI/ML: Implement Adaptive Thresholds to reduce false alarms.

Sensor Fusion: Add Gyroscope and Ambient Sensors for better fall confirmation.

Optimization: Use Edge Computing for better battery life and

integrate with telemedicine.

# REFERENCES

- Academic Papers/Articles Referred

- World Health Organization. "Patient safety." WHO Fact

Sheet.

- Oliver D, Healey F, Haines TP. "Hospital Fall Prevention: A Systematic Review of Implementation, Components, Adherence, and Effectiveness." J Am Geriatr Soc. 2013-03-24. https://pmc.ncbi.nlm.nih.gov/articles/PMC3670303/